Scope of work:

The Contractor shall provide all materials and labor to make improvements to an existing ATV/UTV trail for the Louisiana Department of Wildlife and Fisheries (LDWF).

Location:

Russell Sage WMA

120 Bud Smith Road

Monroe, LA 71202

LDWF Contact

Mitch McGee

318-343-4044

Specifications:

The Contractor shall be responsible for the following:

- ATV/UTV trail is approximately 1 mile in length, 38,213-acres.

- Reshaping road with 4:1 slope

- Provide culvert installation at major drains, reshaping ditches and resurfacing trail with #57 limestone.

- Approximately three 24"x 40' 14-gauge polymer coated corrugated metal pipe culverts

- Cap road with 4” to 6” of #57 limestone, approximately 2500 tons.

- Dozier, track hoe, and road grader shall be provided by the Contractor.

General Information:

- The Contractor shall be responsible for field verifying all dimensions and existing site conditions of work to be performed. The Contractor shall inspect and verify information provided herein on existing materials and equipment, etc. is correct and inform the Project Manager (PM) of any errors or questions prior to commencement of work.

- The Contractor shall take all precautions necessary to prevent damages when delivering materials to site and provide covering to prevent rusting.

- The Contractor shall secure the building, and any sensitive items at the end of each workday in accordance with the designated unit POC.